Loft Conversion  -    Protect your family

The information leaflet “Loft conversions – protect your family” which was published in 2004 highlights mandatory requirements of the Building Regulations for attic conversions including Part B (Fire Safety).

Part B and the accompanying Technical Guidance Document B was amended in 2006. The 2006 amendments supersede those currently printed in the loft conversion leaflet.

The amendments are as follows;

- Point 8a should now read: – ‘unobstructed clear opening of at least 0.33m2 with the width or height being a minimum 450mm. This amends the window dimensions on the diagram.

(i.e. 450mm wide x 733mm high = 0.33m2)’

- Point 9 should now read: – ‘provide mains-powered smoke alarms with battery backup in all circulation areas that form part of the escape route and in all rooms such as kitchens and living rooms that present a high fire risk to give an LD2 system or better to BS 5839 Part 6:2004.  The kitchen should be provided with a heat detector.’

Loft Conversion  -   Protect your family

The information leaflet “Loft conversions – protect your family” which was published in 2004 highlights mandatory requirements of the Building Regulations for attic conversions including Part B (Fire Safety).

Part B and the accompanying Technical Guidance Document B was amended in 2006. The 2006 amendments supersede those currently printed in the loft conversion leaflet.

The amendments are as follows;

- Point 8a should now read: – ‘unobstructed clear opening of at least 0.33m2 with the width or height being a minimum 450mm. This amends the window dimensions on the diagram.

(i.e. 450mm wide x 733mm high = 0.33m2)’

- Point 9 should now read: – ‘provide mains-powered smoke alarms with battery backup in all circulation areas that form part of the escape route and in all rooms such as kitchens and living rooms that present a high fire risk to give an LD2 system or better to BS 5839 Part 6:2004.  The kitchen should be provided with a heat detector.’